Aijia Zhou1*, Nhan Dinh Ngo1,2, Khang Nguyen Duy Lam2, Yile Huang3, Kok-Seng Wong2,4, Haoran Yu5 , Thanh H. Nguyen1,2,4,6

1Department of Civil and Environmental Engineering, The Grainger College of Engineering, University of Illinois at Urbana-Champaign

2VinUni-Illinois Smart Health Center, VinUniversity, Hanoi, Vietnam

3Department of Natural Resources and Environmental Sciences, College of Agricultural, Consumer and Environmental Sciences, University of Illinois Urbana-Champaign

4College of Engineering and Computer Science, VinUniversity, Hanoi, Vietnam

5 Department of Civil and Environmental Engineering, University of Alberta

6Institute of Genomic Biology, University of Illinois at Urbana-Champaign

*Corresponding author: zaijia@illinois.edu; (+1)217-954-3326

## Imbalanced Topic-Based Response Accuracy of Large Language Models in the Indoor Air Quality Domain

### This supplementary file includes 2 tables and one section. Extra supplementary excels are available in the Github: (https://github.com/Nguyen205/LLM_Automatic-evaluation)

Table S1. Topics and the subsequent number of questions related to indoor air quality.

Table S2. Recent representative LLMs used for performance evaluation.

Section 1. Bioaerosols-Related questions and citations

Table S1: Topics and the subsequent number of questions related to indoor air quality.

| Chapter | Topic | # of questions |
| --- | --- | --- |
| Chapter 1 | Fundamentals of Air Quality | 15 |
| Chapter 2 | Properties of Indoor Air Contaminants | 8 |
| Chapter 3 | Particle Size Statistics and Distribution | 8 |
| Chapter 4 | Mechanics of Particles | 13 |
| Chapter 5 | Gas Properties and Kinetics | 11 |
| Chapter 6 | Diffusion and Coagulation of Particles | 12 |
| Chapter 7 | Impaction | 10 |
| Chapter 8 | Sampling Efficiency | 17 |
| Chapter 9 | Deposition, Production, and Resuspension of Airborne Particles | 6 |
| Chapter 10 | Filtration | 7 |
| Chapter 11 | Aerodynamic Air Cleaners | 10 |
| Chapter 12 | Electrostatic Precipitation | 15 |
| Chapter 13 | Control of Gaseous Pollutants | 7 |
| Chapter 14 | Ventilation Requirements and Measurement | 10 |
| Chapter 15 | Ventilation Effectiveness and Air Distribution | 7 |
| Bioaerosol | Bioaerosol-related questions | 18 |

*There are a total of 16 chapters, covering 15 topics of indoor air quality engineering, created from a textbook on indoor air quality (Zhang, 2004), alongside an external topic of bioaerosols from multiple peer-reviewed literature( SI, Section 1).

Table S2. Recent representative LLMs used for performance evaluation.

| Accessibility | Model Name | Size | Context Length | Developer/Series (launch time) |
| --- | --- | --- | --- | --- |
| Closed-source | GPT-4.1 | - | 1.05M | OpenAI/GPT-4 (April 14, 2025) |
|  | Claude 3.7 Sonnet | - | 200K | Anthropic/Claude 3 (March 4, 2024) |
|  | Gemini 2.5 Pro | - | 2M | Google/Gemini 2.5 (February 15, 2024) |
|  | ERNIE-4.5-300B-A47B | 300B | 123K | Baidu/ERNIE 4.5 (Jun 30, 2025) |
| Open weight | Mistral Large 2 | 123B | 131K | Mistral AI (July 24, 2024) |
|  | Llama 4 Scout | 17B | 1.05M | Meta/Llama 4 (April 5, 2025) |
|  | DeepSeek-R1-0528 | 671B | 128K | DeepSeek/DeepSeek-R1 (May 28, 2025) |
|  | Grok 3 | 2,7T | 131K | xAI (April 28, 2025) |
|  | Qwen 3 14B | 14B | 128 K | Alibaba (April 29, 2025) |

Section 1:

### Bioaerosols-Related questions and citations

What are bioaerosols?

Bioaerosols are airborne particulate matter of biological origin (BioPM) with a particle diameter between ∼10 nm and 100 μm in size, with the majority mainly found in the range 0.1–10 μm(Whitby et al.,2022)

How are bioaerosols formed?

Bioaerosols are formed through a variety of processes, including bubble burst, erosion, active and passive dispersion, and mechanical interaction between surfaces(Delort and Amato, 2017)

What are the different types of biological agents found in bioaerosols?

Bioaerosols comprise complex mixtures of airborne microorganisms (i.e. bacteria, fungi, archaea, viruses), the by-products of microbial metabolism, foreign proteins of microorganisms, fragments of cells, endotoxins, mycotoxins, enzymatic proteins, microbial volatile organic compounds (MVOCs), and allergens (CDC).

How do bioaerosols differ from other types of aerosols?

Whether aerosols originated biologically or not (CDC).

How do bioaerosols spread in the environment?

Bioaerosols are easily transported from one place to another because they may remain airborne for long periods of time, facilitating the rapid spread of microorganisms and allowing for their genetic exchange. Three factors dominate aerosol transport, including: 1) aerosol release or emission; 2) dispersion in three dimensions; and 3) deposition rate(Huang et al., 2024)

### Sources and Exposure

What are common sources of bioaerosols in both indoor and outdoor environments?

Microbiological exposures associated with waste can occur indoors, where waste is stored, or outdoors during its collection, and it may be influenced by the process of sorting, transfer or cleaning(Huang et al., 2024).

What are the sources of indoor bioaerosol pollution?

The sources of indoor bioaerosol pollution include outdoor sources (passing through windows, doors, and ventilation); building materials; furnishings; occupants; pets; house plants; and organic wastes Regular or ordinary human activities (e.g., coughing, washing, toilet flushing, talking, walking, sneezing, and sweeping floors) are also capable of generating bioaerosols(Kim, Kabir and Jahan, 2018)

Are there specific professions or environments that have a higher risk of bioaerosol exposure?

Waste management releases airborne microorganisms and bioaerosols into the atmosphere around waste facilities. Consequently, this type of industry and facilities are considered critical in terms of occupational exposure to fungal contamination as they provide optimal conditions for fungal growth. Sources of bioaerosol exposure in occupational activities are diverse enough to include waste sorting and composting, agricultural and food processing activities, the livestock industry, etc(Kim, Kabir and Jahan, 2018).

### Health Impacts and Vulnerability

What are the health risks associated with exposure to bioaerosols, including allergies, asthma, and specific diseases?

Exposure to bioaerosols can lead to a range of deleterious health effects, including allergies, asthma, chronic obstructive pulmonary disease (COPD), and a range of respiratory and infectious diseases, such as pneumonia, influenza, measles, and gastrointestinal illness(Srikanth, Sudharsanam and Steinberg, 2008).

Can bioaerosol be beneficial for human health?

Yes, under certain circumstances, exposure to some microbes is beneficial for health in terms of developing a healthy immune system and protecting children from developing allergies and asthma(Kim, Kabir and Jahan, 2018)

Are certain populations more vulnerable to bioaerosol-related health issues?

Children, the elderly, and immunocompromised individuals are vulnerable to infection by indoor bioaerosols(Kallawicha et al., 2024)

### Measurement and Regulations

How are bioaerosols sampled and analyzed in the air?

The common methods employed for the collection of bioaerosols are impingers, cyclones, impactors, filters, spore traps, electrostatic precipitation, thermal precipitators, condensation traps, and gravitational samplers(Haig et al., 2016). Cultivation methods, microscopy (e.g., fluorescence microscopy, scanning electron microscopy), molecular biological detection (e.g., polymerase chain reaction, DNA sequencing, chemical tracing), immunological detection, spectroscopy, and mass spectrometry are all examples of analytical techniques(Gollakota et al., 2021).

What are the challenges in accurately quantifying bioaerosols?

Accurately quantifying bioaerosols is challenging due to their diverse nature (viable/non-viable, species), low atmospheric concentrations, and significant spatiotemporal variability. Sampling methods can damage or miss particles, and analysis techniques often struggle to differentiate between viable and non-viable organisms, or face issues with detection limits and sample contamination. Lack of standardization also hinders data comparability(Mainelis, 2020)

What are the current regulations or guidelines regarding bioaerosol levels?

Because of limited data and enormous variability in the potential health effects between different types of bioaerosols, standards for bioaerosols based on health risk assessments are still impractical. The Environmental Protection Agency (EPA) of the US and the World Health Organization (WHO) do not have any specific guidelines for bioaerosol concentration levels (WHO, 2009).

The Health and Welfare Department in Canada established the following guidelines: (i) 50 CFU/m3 for one species of fungi warrants immediate investigation; (ii) 100 CFU-m− 3 is unacceptable for the presence of certain fungal pathogens; (iii) 150 CFU/m3 of mixed species is normal; and (iv) up to 500 CFU/m3 should be acceptable if dominated by Cladosporium (WHO (World Health Organization), 1988, EC (Environment Canada), 1989). In the case of Korea, the maximum allowable concentration of total bacterial bioaerosol is 800 CFU/m3 (Ministry of Environment, Republic of Korea, 2010).Based on European literature databases on residential indoor air quality, the residential limit values for airborne fungi, bacteria, and bacterial endotoxin were proposed as 5 × 10^3  CFU/m3, 5 × 10^3  CFU/m− 3, and 5 ng/m3, respectively(Górny and Dutkiewicz, 2002)

### Mitigation and Control

What are some effective methods for controlling or mitigating bioaerosols?

Control technologies can be grouped into three categories: (1) currently implemented technologies; (2) scaled technologies used in industrial and medical settings; and (3) emerging technologies. Category (1) solely consists of fibrous filter media. Category (2) consists of electrostatic precipitators (ESPs), used industrially to collect aerosol particles in higher flow rate systems, and ultraviolet C (UV-C) systems, used in medical settings to inactivate pathogens. Finally, category (3) consists of a variety of technologies, including ionization-based systems, microwaves, and those generating reactive oxygen species, often with the goal of pathogen inactivation in aerosols(Ouyang et al., 2023).

Do air purifiers and HVAC filters effectively capture bioaerosols?

Inexpensive, low-efficiency HVAC filters offer no better particle removal than no filter. WHF(whole house filtration) with high-efficiency HVAC filtration is shown to be more effective in particulate reduction than individual high-efficiency particulate air portable room air cleaners(Sublett, 2011).

How to protect ourselves from bioaerosol exposure?

In general, the following hierarchy of bioaerosol control approaches should be followed: (1) removal of sources, (2) ventilation when source removal is not possible, (3) filtration and air cleaning, and (4) use of personal protective equipment as a last resort(Dannemiller et al., 2025).

### Environmental Factors and Research

How do climate change and human activities influence bioaerosol concentrations and dynamics?

Human activities reduce the environmental quality of the ecosystem, resulting in reduced diversity of microbial communities in bioaerosols, increased presence of allergens and pathogens, and impacts on human health(Feng et al., 2024)

Climate change could reduce airborne bacterial abundance. It may also enhance the survival and spread of viral aerosols and alter pollen seasons, causing earlier peaks. Increased extreme weather events have rapidly changed bioaerosol composition, with heavy precipitation events significantly increasing airborne pathogens like Pseudomonas, Staphylococcus, and Streptococcus, raising inhalation risks(Zhang et al., 2024).

References:

Corinne, W. et al (2022) “Compendium of analytical methods for sampling, characterization and quantification of bioaerosols” Advances in Ecological Research. Elsevier, pp. 101–229. Available at: https://doi.org/10.1016/bs.aecr.2022.09.004.

Dannemiller, K.C. et al. (2025) “Indoor bioaerosols and asthma: Overview, implications, and mitigation strategies,” Journal of Allergy and Clinical Immunology, 155(3), pp. 714–725. Available at: https://doi.org/10.1016/j.jaci.2024.11.027.

Delort, A. and Amato, P. (eds.) (2017) Microbiology of Aerosols. 1st ed. Wiley. Available at: https://doi.org/10.1002/9781119132318.

Feng, X. et al. (2024) “Sources, compositions, spatio-temporal distributions, and human health risks of bioaerosols: A review,” Atmospheric Research, 305, p. 107453. Available at: https://doi.org/10.1016/j.atmosres.2024.107453.

Gollakota, A.R.K. et al. (2021) “Bioaerosols: Characterization, pathways, sampling strategies, and challenges to geo-environment and health,” Gondwana Research, 99, pp. 178–203. Available at: https://doi.org/10.1016/j.gr.2021.07.003.

Górny, R.L. and Dutkiewicz, J. (2002) “Bacterial and fungal aerosols in indoor environment in Central and Eastern European countries,” Annals of agricultural and environmental medicine: AAEM, 9(1), pp. 17–23.

Haig, C.W. et al. (2016) “Bioaerosol sampling: sampling mechanisms, bioefficiency and field studies,” Journal of Hospital Infection, 93(3), pp. 242–255. Available at: https://doi.org/10.1016/j.jhin.2016.03.017.

Huang, Z. et al. (2024) “Bioaerosols in the atmosphere: A comprehensive review on detection methods, concentration and influencing factors,” Science of The Total Environment, 912, p. 168818. Available at: https://doi.org/10.1016/j.scitotenv.2023.168818.

Kallawicha, K. et al. (2024) “Bioaerosols and their health effects,” Health Effects of Indoor Air Pollution. Elsevier, pp. 79–100. Available at: https://doi.org/10.1016/B978-0-443-16090-5.00009-X.

Kim, K.-H., Kabir, E. and Jahan, S.A. (2018) “Airborne bioaerosols and their impact on human health,” Journal of Environmental Sciences, 67, pp. 23–35. Available at: https://doi.org/10.1016/j.jes.2017.08.027.

Mainelis, G. (2020) “Bioaerosol sampling: Classical approaches, advances, and perspectives,” Aerosol Science and Technology, 54(5), pp. 496–519. Available at: https://doi.org/10.1080/02786826.2019.1671950.

Ouyang, H. et al. (2023) “Control technologies to prevent aerosol-based disease transmission in animal agriculture production settings: a review of established and emerging approaches,” Frontiers in Veterinary Science, 10, p. 1291312. Available at: https://doi.org/10.3389/fvets.2023.1291312.

Srikanth, P., Sudharsanam, S. and Steinberg, R. (2008) “BIO-AEROSOLS IN INDOOR ENVIRONMENT: COMPOSITION, HEALTH EFFECTS AND ANALYSIS,” Indian Journal of Medical Microbiology, 26(4), pp. 302–312. Available at: https://doi.org/10.1016/S0255-0857(21)01805-3.

Sublett, J.L. (2011) “Effectiveness of Air Filters and Air Cleaners in Allergic Respiratory Diseases: A Review of the Recent Literature,” Current Allergy and Asthma Reports, 11(5), p. 395. Available at: https://doi.org/10.1007/s11882-011-0208-5.

Zhang, T. et al. (2024) “Effects of extreme precipitation on bacterial communities and bioaerosol composition: Dispersion in urban outdoor environments and health risks,” Environmental Pollution, 344, p. 123406. Available at: https://doi.org/10.1016/j.envpol.2024.123406.

Zhang, Y. (2004) Indoor Air Quality Engineering. 0 ed. CRC Press. Available at: https://doi.org/10.1201/b12485.